METAM RV Metallographic Inverted Microscope

METAM RV microscopes with upper location of the stage are based on one unified stand, differing in configuration of furnished assemblies: set of objectives, eyepieces and other accessories. Various configuration options allow customers to choose the microscope in accordance with the specific character of application: metallographic microscopes – operational METAM RV-21, METAM RV-22, student-oriented METAM RV-23 are designed for visual examination of microstructure of metals, alloys and other non-transparent objects in reflected light with perpendicular bright-field and dark-field illumination, as well as for studying objects in polarized light and by differential interference-contrast (DIC) illumination method. Specifications METAM RV-21; METAM RV-22; METAM RV-23. Magnification; 80-1000; 80-1000; 80-507. Magnification of eyepieces; 10;12.5;20; 10; 12.5; 20; 10; 12.5. Objectives; On turret F=25mm, A=0.17mm; F=16mm, A=0.30mm; F=6.3mm, A=0.60mm; F=4.0mm, A=0.85mm;; On turret F=25mm, A=0.17mm; F=6.3mm, A=0.60mm; F=4.0mm, A=0.85mm;; On turret F=25mm, A=0.17mm; F=6.3mm, A=0.60mm;. Objectives with DIC module; F=25mm, A=0.17mm; F=16mm, A=0.30mm; F=6.3mm, A=0.60mm;. Range of stage displacement, mm; 70x50; 70x50; 70x50. Photography capability Photographic attachment; 24x36mm (optional). Microscope dimensions, mm; 310x280x270; 310x280x270; 310x280x270. Mass, kg; 8; 7; 6.5..